# DWDM ITU Channels Guide

DWDM ITU Channels Overview

ITU G.694.1 standard DWDM region is from 1528.77nm to 1563.86nm that resides mostly within the C band. DWDM can have 100GHz (0.8 nm) wavelength spacing for 40 channels, or 50GHz (0.4 nm) spacing for 80 channels. The complete channel grid for 100GHz DWDM is shown below.

| Channel | Frequency (THz) | Center Wavelength (nm) | Channel | Frequency (THz) | Center Wavelength (nm) |
| --- | --- | --- | --- | --- | --- |
| C17 | 191.7 | 1563.86 | C40 | 194 | 1545.32 |
| C18 | 191.8 | 1563.05 | C41 | 194.1 | 1544.53 |
| C19 | 191.9 | 1562.23 | C42 | 194.2 | 1543.73 |
| C20 | 192 | 1561.41 | C43 | 194.3 | 1542.94 |
| C21 | 192.1 | 1560.61 | C44 | 194.4 | 1542.14 |
| C22 | 192.2 | 1559.79 | C45 | 194.5 | 1541.35 |
| C23 | 192.3 | 1558.98 | C46 | 194.6 | 1540.56 |
| C24 | 192.4 | 1558.17 | C47 | 194.7 | 1539.77 |
| C25 | 192.5 | 1557.36 | C48 | 194.8 | 1538.98 |
| C26 | 192.6 | 1556.55 | C49 | 194.9 | 1538.19 |
| C27 | 192.7 | 1555.75 | C50 | 195 | 1537.4 |
| C28 | 192.8 | 1554.94 | C51 | 195.1 | 1536.61 |
| C29 | 192.9 | 1554.13 | C52 | 195.2 | 1535.82 |
| C30 | 193 | 1553.33 | C53 | 195.3 | 1535.04 |
| C31 | 193.1 | 1552.52 | C54 | 195.4 | 1534.25 |
| C32 | 193.2 | 1551.72 | C55 | 195.5 | 1533.47 |
| C33 | 193.3 | 1550.92 | C56 | 195.6 | 1532.68 |
| C34 | 193.4 | 1550.12 | C57 | 195.7 | 1531.9 |
| C35 | 193.5 | 1549.32 | C58 | 195.8 | 1531.12 |
| C36 | 193.6 | 1548.51 | C59 | 195.9 | 1530.33 |
| C37 | 193.7 | 1547.72 | C60 | 196 | 1529.55 |
| C38 | 193.8 | 1546.92 | C61 | 196.1 | 1528.77 |
| C39 | 193.9 | 1546.12 |  |  |  |

HOTCOM DWDM Transceiver Modules Quick View

HOTCOM CWDM transceivers are available with all 44 DWDM wavelengths, including DWDM SFP, DWDM SFP+, DWDM XFP and Tunable DWDM transceivers that support transmission distance up to 100km. Tunable DWDM transceivers is able to support a specific channel in a DWDM optical network, allowing for remotely change wavelengths in software.

- 40KM DWDM Modules

DWDM SFP 40KM
DWDM SFP+ 40KM
DWDM XFP 40KM

- 80KM DWDM Modules

DWDM SFP 80KM
DWDM SFP+ 80KM
Tunable DWDM SFP+ 80KM
DWDM XFP 80KM
Tunable DWDM XFP 80KM

- 100KM DWDM Modules

DWDM SFP 100KM

- HOTCOM DWDM Mux/Demux Solution

DWDM is used in long haul networks to help alleviate fiber exhaustion and the costs involved in running new fiber. Each DWDM channel can transport up to 100G, and distances over 1000 kilometers can be achieved with the use of optical amplifiers.

| Product Description | Line Type | Wavelength |
| --- | --- | --- |
| 8CH DWDM Mux/Demux | Dual Fiber | C21-C28, C53-C60 |
|  | Single Fiber | C21-C35, C22-C36 |
| 16CH DWDM Mux/Demux | Dual Fiber | C17-C32, C21-C36, C27-C42, C43-C58 |
| 40CH DWDM Mux/Demux | Dual Fiber | C21-C60 |
| 96CH DWDM Mux/Demux | Dual Fiber | C15-C62.5 |