Fiber Optic Innovator

CWDM OADM Module (OADM)

| Features |  |
| --- | --- |
| Low insertion loss High channel isolation High stability and reliability |  |
| Application |  |
| CWDM system Metro/Access Networks CATV Fiberoptic System |  |

Specifications

| Type Parameter |  |  | 1ch | 2ch |
| --- | --- | --- | --- | --- |
| Channel Wavelength (nm) |  |  | 1471, 1491,..1551, 1571, 1591, 1611 |  |
| Channel space (nm) |  |  | 20 |  |
| Channel bandwidth (nm) |  |  | λc ± 6.5 |  |
| Add/Drop Channel Ripple (dB) |  |  | ≤0.4 |  |
| Insertion loss (dB) | Add/Drop Ch. |  | ≤0.7 | ≤1.0 |
|  | Express Ch. |  | ≤0.6 | ≤1.2 |
| Isolation (dB) | Drop | Adjacent Ch | ≥30 |  |
|  |  | Non-adjacent Ch | ≥40 |  |
|  | Add | Adjacent Ch | NA |  |
|  |  | Non-adjacent Ch | NA |  |
| Express Channel Isolation (dB ) |  |  | ≥25 |  |
| Directivity (dB) |  |  | ≥55 |  |
| Return loss (dB) |  |  | ≥50 |  |
| PDL (dB) |  |  | ≤0.15 |  |
| Wavelength thermal stability (nm/℃) |  |  | ≤0.003 |  |
| Insertion loss thermal stability (dB/℃) |  |  | ≤0.005 |  |
| Power handling (mW) |  |  | ≤500 |  |
| Operating temperature (℃) |  |  | 0 ~ +70 |  |
| Storage temperature (℃) |  |  | -40 ~ +85 |  |
| Dimensions (mm) |  |  | 80x60x10 or 100x80x10 |  |

Ordering Information:

| OADM | Type | Port Type | Wavelength | Pigtail Type | Fiber Type | Length | Connector |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | C=CWDM | 1=1ch 2=2ch | 1471=1471 1491=1291 …… 1611=1611 | 900=900um loose tube 2000=2mm loose tube 3000=3mm loose tube | 1=SMF-28e | 1= 1m X=Specif y | NE=None FA=FC/APC FC=FC/UPC SA=SC/APC SC=SC/UPC LC=LC/UPC LA=LC/APC XX=Other |

www.htopto.com