Fiber Optic Innovator

Coarse Wavelength Division Multiplexer Module (CWDM)

| Features |  |
| --- | --- |
| Low insertion loss High channel isolation High stability and reliability |  |
| Application |  |
| CWDM system Metro/Access Networks CATV Fiberoptic System |  |

Specifications

| Parameter |  | 4 Channel | 8 Channnel | 16 Channel |
| --- | --- | --- | --- | --- |
| Central Wavelength(nm) |  | 1271, 1291, 1311, …..1551, 1571, 1591, 1611 |  |  |
| Channel space (nm) |  | 20 |  |  |
| Channel bandwidth (nm) |  | λc ± 6.5 |  |  |
| Channel flatness (dB) |  | ≤0.4 |  |  |
| Insertion Loss (dB) Max |  | ≤2.0 | ≤3.0 | ≤5.0 |
| Demux Isolation (dB) | Adjacent channel | ≥30 |  |  |
|  | Non-adjacent channel | ≥40 |  |  |
| Mux Isolation (dB) | Adjacent channel | ≥15 |  |  |
|  | Non-adjacent channel | ≥15 |  |  |
| Channel uniformity（dB） |  | ≤1.0 |  |  |
| Directivity (dB) |  | ≥55 |  |  |
| Return loss (dB) |  | ≥50 |  |  |
| PDL (dB) |  | ≤0.15 |  | ≤0.20 |
| Wavelength thermal stability (nm/℃) |  | ≤0.003 |  |  |
| Insertion loss thermal stability (dB/℃) |  | ≤0.005 |  |  |
| Power handling (mW) |  | ≤500 |  |  |
| Operating temperature (℃) |  | 0 ~ +70 |  |  |
| Storage temperature (℃) |  | -40 ~ +85 |  |  |
| Dimensions (LxWxH) (mm) |  | 100x80x10(ABS) |  | 141x115x18(ABS) |
|  |  | 110x95x7.5(Metal) |  |  |

Ordering Information:

| CWDM | Type | Port Type | Wavelength | Pigtail Type | Fiber Type | Length | Connector |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | M=Mux D=Demux | 4=4ch 8=8ch 16=16ch | 1271=1271 1291=1291 …… 1611=1611 | 900=900um loose tube 2000=2mm loose tube 3000=3mm loose tube | 1=SMF-28e | 1= 1m X=Specify | NE=None FA=FC/APC FC=FC/UPC SA=SC/APC SC=SC/UPC LC=LC/UPC LA=LC/APC XX=Other |

www.htopto.com