Fiber Optic Innvator

Compact CWDM Module (Mini CWDM)

| Features |  |
| --- | --- |
| Low insertion loss & High channel isolation High stability and reliability |  |
| Application |  |
| CWDM system Metro/Access Networks CATV Fiberoptic System |  |

Specifications

| Parameter |  | 4 Channels | 8 Channels |
| --- | --- | --- | --- |
| CWDM Channel Insertion Loss (dB) |  | ≤1.0 (Typ 0.8) | ≤1.5 (Typ 1.0) |
| Upgrade channel Insertion Loss (dB) (Optional) |  | ≤1.0 | ≤1.5 |
| Central wavelength (nm) |  | 1471, 1491, 1511, 1531, 1551, 1571, 1591, 1611 |  |
| Channel space (nm) |  | 20 |  |
| Channel bandwidth (nm) |  | λc ± 6.5 |  |
| Isolation (dB) | adjacent channel | ≥30 |  |
|  | non-adjacent channel | ≥45 |  |
| Directivity (dB) |  | ≥55 |  |
| Return loss (dB) |  | ≥45 |  |
| PDL (dB) |  | ≤0.20 |  |
| PMD (ps) |  | ≤0.20 |  |
| Power handling (mW) |  | ≤300 |  |
| Operating temperature (℃) |  | 0 ~ +70 |  |
| Storage temperature (℃) |  | -40 ~ +85 |  |
| Dimensions (mm) |  | 50x28.8x6.5 |  |

Package Dimensions:

Ordering Information:

| MCWDM | Type | Port Type | Wavelength | Pigtail Type | Fiber Type | Length | UpgradeType | Connector |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | M=Mux D=Demux | 4=4ch 8=8ch | 1471 1491 …… 1611 | 250=250um 900=900um | 1=SMF-28e | 1= 1m | WO=W/O UPG Ch W=With UPG Ch | NE=None FA=FC/APC FC=FC/UPC SA=SC/APC SC=SC/UPC LC=LC/UPC LA=LC/APC XX=Other |

www.htopto.com