Fiber Optic Innovator

200Ghz Dense Wavelength Division Multiplexer (DWDM)

| Features |  |
| --- | --- |
| Low Insertion Loss High Channel Isolation High Stability and Reliability Epoxy Free Optical Path |  |
| Application |  |
| DWDM Network Wavelength Routing Fiber Optical Amplifier CATV Fiberoptic System |  |

Specifications

| Parameter |  | 1x2 200Ghz DWDM |
| --- | --- | --- |
| Channel Wavelength (nm) |  | 1529.55~1561.42 (ITU 20~ 60) |
| Center Wavelength Accuracy (nm) |  | ± 0.1 |
| Channel Passband (@-0.5dB Bandwidth) (nm) |  | ≥ 0.5 |
| Insertion Loss (dB) | Add/Drop Ch. | ≤ 0.9 |
|  | Express Ch. | ≤0.4 |
| Channel Ripple (dB) |  | ≤ 0.4 |
| Isolation(dB) | Adjacent Ch | ≥ 30 |
|  | Non-adjacent Ch | ≥40 |
| Express Channel Isolation (dB) |  | ≥ 12 |
| Insertion Loss Temperature Sensitivity (dB/°C ) |  | < 0.003 |
| Wavelength Temperature Shifting (nm/°C ) |  | < 0.002 |
| Polarization Dependent Loss (dB) |  | ≤ 0.10 |
| Polarization Mode Dispersion (ps) |  | ≤ 0.1 |
| Directivity (dB) |  | ≥ 50 |
| Return Loss (dB) |  | ≥45 |
| Power Handling (mW) |  | ≤300 |
| Operating Temperature (°C) |  | 0 ~ +70 |
| Storage Temperature (°C) |  | -40 ~ +85 |
| Package Dimension (mm) |  | Ø5.5 x L34 |

Ordering Information

| DWDM | Channel Type | Spacing | ITU Channel | Pigtail Type | Fiber Type | Length | Connector |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1=1ch | 2=200Ghz | 21=21ch 22=22ch ………… 60=60ch | 250=250um Bare Fiber 900=900um Loose Tube 2000=2mm Loose Tube 3000=3mm Loose Tube | 1=SMF-28e | 1=1m X=Specify | NE=None FA=FC/APC FC=FC/UPC SA=SC/APC SC=SC/UPC LC=LC/UPC LA=LC/APC MU=MU/UPC XX=Other |

Tel: +86 755 8619763                                                  www.htopto.com                                     sales@htopto.com