Fiber optic innovator

100Ghz Optical Add/Drop Module (OADM)

| Features |  |
| --- | --- |
| Low Insertion Loss High Channel Isolation High Stability and Reliability |  |
| Application |  |
| DWDM Network Wavelength Routing Fiber Optical Amplifier CATV Fiberoptic System |  |

Specifications

| Parameter |  |  | 1ch | 2ch |
| --- | --- | --- | --- | --- |
| Channel Wavelength (nm) |  |  | 1530.33 ~ 1560.61 (21~ 59 ITU grid) |  |
| Center Wavelength Accuracy (nm) |  |  | ± 0.05 |  |
| Minimum Channel Spacing (GHz) |  |  | 100 |  |
| Channel Passband (@-0.5dB bandwidth) (nm) |  |  | > 0.22 |  |
| Insertion Loss (dB) |  | Add / Drop Ch1 | ≤1.2 | ≤1.2 |
|  |  | Add / Drop Ch 2 | - | ≤1.5 |
|  |  | Express Ch. | ≤1.0 | ≤1.4 |
| Add / Drop Channel Ripple (dB) |  |  | ≤0.3 |  |
| Isolation (dB) | Demux | Adjacent Ch. | ≥ 25 |  |
|  |  | Non-adjacent Ch. | ≥40 |  |
|  | Mux | Adjacent Ch. | N/A |  |
|  |  | Non-adjacent Ch. | N/A |  |
| Express Channel Isolation (dB) |  |  | > 25 |  |
| Insertion Loss Temperature Sensitivity (dB/°C ) |  |  | < 0.003 |  |
| Wavelength Temperature Shifting (nm/°C ) |  |  | < 0.002 |  |
| Polarization Dependent Loss (dB) |  |  | < 0.10 |  |
| Polarization Mode Dispersion (ps) |  |  | < 0.1 |  |
| Directivity (dB) |  |  | ≥50 |  |
| Return Loss (dB) |  |  | ≥ 45 |  |
| Power Handling (mW) |  |  | 300 |  |
| Operating Temperature (°C) |  |  | 0 ~ +70 |  |
| Storage Temperature (°C) |  |  | -40 ~ +85 |  |
| Package Dimension (mm) |  |  | 100x80x10 |  |

* channel cascading sequence can be adjusted upon requests.

www.htopto.com

Fiber optic innovator

Optical Path Diagram

| λ1 Drop |  | COM |
| --- | --- | --- |
| λ2 Drop |  |  |
| λ1 Add |  |  |
| λ2 Add |  |  |
|  |  | EXP |

Ordering Information:

| HT-OADM | Spacing | Channel Type | 1st ITU Channel | Pigtail Type | Fiber Type | Length | Connector |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1=100Ghz | 1=1ch 2=2ch | 21=21ch 22=22ch …… 60=60ch | 900=900um loose tube 2000=2mm loose tube 3000=3mm loose tube | 1=SMF-28e | 1= 1m X=Specify | NE=None FA=FC/APC FC=FC/UPC SA=SC/APC SC=SC/UPC LC=LC/UPC LA=LC/APC XX=Other |

www.htopto.com