| ① | Channel Space | 1=100GHz; 2=200GHz; |
| --- | --- | --- |
| ② | Channel Type | 1CH; |
| ③ | ITU Grid | 21=21ch; 21*=21*ch; 22=22ch; 22*=22*ch; …; 72*=72*ch; |
| ④ | Pigtail Type | 250=250um Bare Fiber; 900=900μm loose tube; 3000=3mm Cable; |
| ⑤ | Fiber Type | 1=SMF-28e; |
| ⑥ | Fiber Length | 1=1m; |
| ⑦ | Connector | NE=None; FA=FC/APC; FC=FC/UPC; SA=SC/APC; SC=SC/UPC; LC=LC/UPC; XX=Others; |
| ⑧ | Package | 100x80x10; LGX; |

Fiber optic innovator	_

High Isolation Dense Wavelength Division Multiplexer (DWDM)

Features	Applications

High Demux Channel Isolation	Long Haul DWDM System

Low Polarization Dependent Loss	Metro DWDM System

Flat and Wide Passband	Access/Enterprise Network

High Reliability	Test Instrument

C34	1550.12

Specifications	C15	1565.50

| Parameters |  | Unit | 100G | 200G |
| --- | --- | --- | --- | --- |
| Channel Wavelength CWL |  | nm | ITU Grid |  |
| Center Wavelength Accuracy |  | nm | ±0.05 |  |
| Channel Passband (@-0.5dB Bandwidth) |  | nm | ≥0.22 | ≥0.50 |
| Insertion Loss | Add / Drop CH | dB | ≤1.0 |  |
|  | Express CH | dB | ≤1.0 |  |
| Isolation | Add / Drop CH | dB | ≥40 |  |
|  | Express CH | dB | ≥30 for adj Channel, ≥40 for non-adj Channel |  |
| Insertion Loss Temperature Sensitivity |  | dB/°C | ≤0.003 |  |
| Wavelength Temperature Shifting |  | nm/°C | ≤0.002 |  |
| PDL |  | dB | ≤0.30 |  |
| PMD |  | ps | ≤0.10 |  |
| Directivity |  | dB | ≥50 |  |
| Return Loss |  | dB | ≥45 |  |
| Fiber Type |  |  | SMF-28e |  |
| Fiber Length |  | m | 1.0 |  |
| Power Handling |  | mW | ≤300 |  |
| Operating Temperature |  | ℃ | -0 ~ +70 |  |
| Storage Temperature |  | ℃ | -40 ~ +85 |  |
| Dimensions |  | mm | 100x80x10 |  |

* Insertion Loss +0.2dB, and Return Loss -5dB if add connector.

* RoHS Compliant. Package Dimensions

Ordering Information

HT-DWDM- ①-②②②-③③-④④④-⑤-⑥-⑦⑦-⑧⑧⑧⑧⑧⑧⑧⑧⑧

_____________________________

http://www.htopto.com